Safe Operating Procedures for Generators

Purpose

To define the safe operating procedures in a manner that informs and instructs employees of [Employer/Organization Name] on the key health and safety hazards and controls to remember when using the portable generator.

Background

The following hazards may occur when using the generator:

- Burns

- Fire or explosion

Carbon monoxide poisoning

Electric shock

Equipment or property damage

Personal Protective Equipment

- Safety footwear

Hearing protection

Safe Operating Procedure

- Complete a pre-use inspection. If any defects are noted, the equipment must be removed from service and the supervisor must be notified immediately to ensure equipment is repaired.

- Operators must have read and understood the operator’s manual.

- Ensure operator’s manual for equipment is available to operators.

- Excavators are to be used for the manufacturer’s designed purpose only

- Ensure guards and shields are firmly in place and in good condition. Do not operate the equipment without appropriate guards, shields, plates and other safety protective devices in place.

- Ensure safety decals are legible, order replacements if they are not.

Confirm the generator’s output is sufficient for the device(s) you wish to connect.

Do not operate the generator near gasoline or gaseous fuel.

Do not fill the fuel tank while the engine is running.

Do not smoke or use open flames near the fuel tank.

Be careful not to spill fuel during refueling, if a spill occurs, wipe off and let dry before starting the engine.

Do not place any flammable materials or items (e.g. fuel, matches, gunpowder, oily cloths, straw, trash, etc) near the generator.

Always operate the generator in a well-ventilated area, do not operate inside a room, cave, tunnel or other insufficiently ventilated area.

If it must be used indoors, the area must be well-ventilated and extreme caution must be taken regarding the discharge of exhaust gases.

Do not enclose the generator, or cover with a box while it is running.

Keep the generator at least 1 metre (3 feet) away from any structure or building during use.

The generator must be operated on a level surface.

Be aware of the wiring or extension cord placement from the generator to the connecting device. If the wire is under the generator, or in contact with a vibrating part, it may cause damage.

Use only ‘listed’ extension cords.

When a tool or appliance is used outdoors, only use extension cords marked for outdoor use.

Replace damaged or worn cords immediately.

Store extension cords in a dry and well-ventilated area when not in use.

Confirm all necessary electrical grounding procedures are followed during each use.

Do not connect the generator to a commercial power line – this may short–circuit the generator and cause an electric shock hazard. Use the transfer switch for connecting to a domestic circuit.

Do not operate in rain, wet or damp conditions, or with wet hands. If wet, wipe and dry it well before starting.

Do not touch the engine or muffler area for some time after operation, it will be extremely hot.

Keep children and all bystanders at a safe distance from work areas.

Always switch off the generator’s AC circuit breaker and disconnect devices when not in use, and before servicing, adjusting, or installing accessories and attachments.

The engine must be stopped before starting any maintenance, servicing or repair.

Fuelling must be done outdoors and while the equipment is off.

Repairs to the equipment must be performed by qualified personnel, using original equipment manufacturer (OEM) parts or equivalent.

Additional Resources

SOP for Fuelling

Document Management

| Effective Date: |  |
| --- | --- |
| Revision Date: |  |